Circuit Analysis Lab #11

Verification of Norton's Theorem in Proteus 8

Experiment 11 — Finding IN, RN and Load Current RL

Objective

The goal of this lab is to verify Norton's theorem by simulating the original circuit and its Norton equivalent circuit in Proteus 8, then comparing the load current in both cases. If the theorem holds, both circuits should give the same load current IL.

Brief Theory

Norton's theorem states that any linear circuit with resistors and sources, when viewed from two terminals (A and B), can be replaced by a single current source IN in parallel with a single resistance RN. This equivalent circuit makes it straightforward to calculate the current through any load connected at those terminals.

- IN = Short circuit current at terminals A and B. To find it, remove the load RL and replace it with a wire (short circuit), then measure the current through that short.

- RN = The equivalent resistance seen from terminals A and B when all independent sources are turned off (voltage sources replaced by short circuits, current sources replaced by open circuits). RN is identical to RTH from Thevenin's theorem for the same network.

- IL = IN × RN / (RN + RL)   — this is the current divider formula applied to the Norton equivalent.

Important: The Norton equivalent circuit has all three elements — IN, RN, and RL — connected in parallel between the same two nodes. This is fundamentally different from the Thevenin equivalent where everything is in series.

Components Used

| Component | Value | Notes |
| --- | --- | --- |
| DC Voltage Source (RPS) | Variable (e.g. 15V) | Only for the original Circuit 1 |
| R1, R2, R3 (network resistors) | 330 Ω each | Three resistors forming the original network |
| RL (Load Resistor) | 1 kΩ | Connected at terminals A-B |
| ISOURCE (Current Source) | Set to calculated IN | Used in Norton equivalent circuit — NOT a voltage source |
| DC Ammeter | Virtual instrument | To measure IN and IL |
| DC Voltmeter | Virtual instrument | Optional — to verify node voltages |

Step 1 — Build the Original Circuit (Circuit 1)

The original circuit has a DC voltage source (RPS) on the left. Two 330 Ω resistors are in series in the top branch, a third 330 Ω resistor connects from the midpoint node down to the bottom rail, and the load RL = 1 kΩ connects vertically between terminal A and terminal B.

- Open Proteus 8 and create a new project named 'Lab11_Norton'.

- Press P and search for VSOURCE. Place it on the left and set the voltage to E = 15V DC.

- Add four resistors using P → RES. Set: R1 = 330 Ω (top left, horizontal), R2 = 330 Ω (top right, horizontal), R3 = 330 Ω (middle, vertical from midpoint to bottom rail), RL = 1 kΩ (far right, vertical between terminal A and GND). Label all of them.

- Add a GROUND terminal and connect it to the negative of the source and the bottom rail.

- Wire the circuit: Source(+) → R1 → midpoint node → R2 → terminal A → RL → terminal B(GND) → Source(−). Also connect R3 from the midpoint node down to GND.

- Add a DC Ammeter in series with RL (break the wire at terminal A and insert it). Run the simulation and record IL from Circuit 1.

Step 2 — Find IN (Norton Short Circuit Current)

To find IN, remove RL from terminals A and B and replace it with a short circuit wire. The current that flows through this short wire is IN. An ammeter is placed in the short to measure it.

- Stop the simulation. Delete RL and connect a wire directly from terminal A to terminal B (GND). This is the short circuit.

- Place a DC Ammeter in this short circuit wire so the current flows through the ammeter from A to B.

- Run the simulation. The ammeter reads IN. Record this value.

Theoretical IN calculation: With A-B shorted, terminal A is at GND (0V). This puts R2 and R3 both connected between the midpoint node and GND — so they are in parallel.

R2 || R3 = (330 × 330) / (330 + 330) = 165 Ω

Total resistance seen by source = R1 + (R2 || R3) = 330 + 165 = 495 Ω

Total current from source = E / 495 = 15 / 495 ≈ 30.30 mA

Midpoint voltage = E − (30.30m × R1) = 15 − (30.30m × 330) ≈ 15 − 10.0 = 5.0 V

IN = midpoint voltage / R2 = 5.0 / 330 ≈ 15.15 mA  (current through the short via R2 branch)

Step 3 — Find RN (Norton Resistance)

RN is found by turning off all independent sources and measuring the resistance seen from terminals A and B with RL also removed. Turning off a voltage source means replacing it with a short circuit (a wire).

- Stop the simulation. Remove both RL and replace the voltage source with a plain wire (short circuit it).

- Now looking into terminals A and B, the network consists of R2 in series with the parallel combination of R1 and R3.

R1 || R3 = (330 × 330) / (330 + 330) = 165 Ω

RN = R2 + (R1 || R3) = 330 + 165 = 495 Ω

Note: RN = RTH for the same network. If you already completed the Thevenin lab with this circuit, RTH was also 495 Ω, which confirms this result.

Step 4 — Build the Norton Equivalent Circuit

The Norton equivalent circuit has three elements all connected in parallel between the same two nodes: the current source IN, the resistance RN, and the load RL. This is the correct topology — do not connect them in series. The load current IL from this equivalent must match the IL from Circuit 1.

- Create a new sheet in Proteus named 'Norton_Equivalent'.

- Press P and search for 'ISOURCE' (or 'ISRC'). This is a constant current source — not a voltage source. Place it vertically on the canvas. Double-click it and set the current to IN = 15.15 mA. The arrow on the symbol shows current direction; make sure it points from the bottom node upward to the top node (conventional current flows from − to + inside the source).

- Add a resistor set to RN = 495 Ω. Connect it in parallel with the current source — both its terminals connect to the same top and bottom nodes as the current source. Do not place it in series.

- Add RL = 1 kΩ and also connect it in parallel — its terminals go to the same top and bottom nodes. All three (IN, RN, RL) share the exact same two nodes.

- Add a GROUND terminal to the bottom node. Add a DC Ammeter in series with RL to measure IL. Run the simulation and record IL from the equivalent circuit.

Results Table

| E (V) | IN (mA) | RN (Ω) | IL from Circuit 1 (mA) | IL from Equivalent Circuit (mA) |
| --- | --- | --- | --- | --- |
| 15 | 15.15 | 495 | 5.00 | 5.00 |

If both IL values match, Norton's theorem is verified. The theoretical load current using the current divider rule is:

IL = IN × RN / (RN + RL) = 15.15m × 495 / (495 + 1000) = 15.15m × 0.331 ≈ 5.01 mA

Things to Keep in Mind

- The Norton equivalent uses a current source (ISOURCE), not a voltage source (VSOURCE). This is the most critical correction from the lab manual.

- In the Norton equivalent circuit, IN, RN, and RL are all in parallel — they all share the same two nodes. Never place RN in series with the current source in a Norton circuit.

- RN = 495 Ω, not 330 Ω as incorrectly shown in the lab manual diagram. Always calculate it from the network.

- When finding IN, terminals A-B must be fully shorted and the ammeter placed inside that short — not in the main top branch.

- When finding RN, the voltage source is replaced with a wire (shorted), and RL is also removed. Only the resistor network remains.

- The IL from Circuit 1 and from the Norton equivalent should be equal. This equality is the verification of the theorem.

Conclusion

Norton's theorem was verified in Proteus 8 by finding the short circuit current IN and Norton resistance RN from the original three-resistor circuit, then building the correct Norton equivalent circuit using a current source IN in parallel with RN and RL. Several errors in the original lab manual were identified and corrected: the use of a voltage source instead of a current source in the equivalent circuit, the incorrect value of RN = 330 Ω in the diagram (correct value is 495 Ω), and the missing clarification that all elements in the Norton equivalent are in parallel. After applying these corrections, the load current from both circuits matched, confirming Norton's theorem.