University of Engineering & Technology

Lahore

Circuit Analysis

Lab #2

Constructing Circuits in Proteus

Submitted By:

Shifa Ikram – 2025-CE-68

Submitted To:

Sir Mehmood

For the fulfillment of,

Circuit Analysis Lab

DERARTMENT OF COMPUTER ENGINEERING

UNIVERSITY OF ENGINEERING AND TECHNOLOGY, LAHORE

Waveform Measurements and Basic Resistive Circuits

Objective

The purpose of this report is to explain how the two circuits from Lab #2 can be built and simulated using Proteus 8 software. Proteus lets you draw a circuit on screen and then run a simulation to see voltages and waveforms just like you would on a real oscilloscope. This is useful for verifying your results before or after doing the actual hardware experiment.

Brief Introduction to Proteus 8

Proteus 8 is an electronic simulation software. The main window is called ISIS (schematic editor) where you draw the circuit.

Experiment #1 — Circuit A

Circuit Description

Circuit A has a sinusoidal voltage source V1 (5V peak-to-peak, 100 Hz) connected to two resistors R1 = 5.1 kΩ and R2 = 3.3 kΩ in series. It is basically a voltage divider. The oscilloscope is used to measure the input voltage, voltage across R1, and voltage across R2.

| Component | Value | Notes |
| --- | --- | --- |
| V1 (Voltage Source) | 5 Vpp, 100 Hz, Sine | In Proteus, set Amplitude = 2.5 V (not 5 V) |
| R1 | 5.1 kΩ | Series resistor |
| R2 | 3.3 kΩ | Series resistor |
| Oscilloscope | Virtual (built-in) | To measure waveforms |

How to Build Circuit A in Proteus

- Open Proteus 8 and go to File → New Project. Give it a name like 'CircuitA' and click Finish to open a blank schematic.

- Press P to open the component search. Type 'VSINE' and select it from the list. Place it on the left side of the canvas. Double-click it and set: Amplitude = 2.5 V, Frequency = 100 Hz. (Important: Proteus uses peak voltage, so for 5 Vpp you enter 2.5 V.)

- Press P again and search for 'RES'. Place two resistors. Double-click the first one, set its value to 5.1k and label it R1. Do the same for the second one, set 3.3k and label it R2. Place R1 to the right of V1 and R2 below R1.

- Click the Terminals Mode button on the left toolbar and select GROUND. Place it at the bottom of the circuit to connect the negative terminal of V1 and the bottom of R2 to ground.

- Press W to start drawing wires. Connect: V1 positive → top of R1 → bottom of R1 → top of R2 → bottom of R2 → GND. Also connect V1 negative to GND. This completes the series loop.

- Click 'Virtual Instruments Mode' on the left toolbar and place an Oscilloscope on the canvas. Connect Channel A to the input node (positive of V1). Connect Channel B to the midpoint between R1 and R2 (this measures voltage across R2 since the other end is GND). Connect oscilloscope GND to circuit GND.

- Press the green Play button to run the simulation. The oscilloscope window opens. Adjust Volts/div and Time/div to see the waveforms clearly. For 100 Hz, set Time/div to around 2 ms/div so you can see at least 2 full cycles.

- Read off the peak-to-peak voltages from the screen for the input, V_R1, and V_R2. To get V_R1, subtract V_ChB from V_ChA since both channels show voltage with respect to GND.

Expected Results for Circuit A

Using the voltage divider formula:

Total resistance = 5.1k + 3.3k = 8.4 kΩ

V_R1 (peak) = 2.5 × (5100 / 8400) ≈ 1.52 V   →   Vpp ≈ 3.04 V

V_R2 (peak) = 2.5 × (3300 / 8400) ≈ 0.98 V   →   Vpp ≈ 1.96 V

RMS values = peak / √2, so V_R1(rms) ≈ 1.07 V and V_R2(rms) ≈ 0.69 V

Experiment #2 — Circuit B

Circuit Description

Circuit B is a bit more complex. It has three resistors — R1 = 4.7 kΩ, R2 = 3.3 kΩ, and R3 = 5.1 kΩ — connected in a loop with the voltage source V1 (7 Vpp, 200 Hz). Looking at Figure 7 in the lab manual, R1 is at the top, R2 is on the right side (vertical), and R3 is at the bottom. So R1 and R3 are in series in the loop and R2 bridges across the middle section. The oscilloscope measures voltage across R1, R2, R3, R1+R2, and R2+R3.

| Component | Value | Notes |
| --- | --- | --- |
| V1 (Voltage Source) | 7 Vpp, 200 Hz, Sine | Set Amplitude = 3.5 V in Proteus |
| R1 | 4.7 kΩ | Top horizontal resistor |
| R2 | 3.3 kΩ | Right vertical resistor |
| R3 | 5.1 kΩ | Bottom horizontal resistor |
| Oscilloscope | Virtual (built-in) | Multiple probe points needed |

How to Build Circuit B in Proteus

- Create a new project (or add a new sheet) in Proteus and open a blank schematic.

- Press P and add a VSINE source. Double-click and set Amplitude = 3.5 V (half of 7 Vpp), Frequency = 200 Hz. Place it on the left side of the canvas, oriented vertically.

- Add three resistors using P → RES. Set R1 = 4.7k (place horizontally at the top), R2 = 3.3k (place vertically on the right side — right-click to rotate before placing), R3 = 5.1k (place horizontally at the bottom). Label them R1, R2, R3.

- Add a GROUND terminal from the Terminals Mode and place it at the bottom-left — this is where V1 negative and the left end of R3 both connect.

- Press W and wire the circuit: V1(+) → left end of R1 → right end of R1 connects to top of R2 (call this Node A). Bottom of R2 connects to right end of R3 (call this Node B). Left end of R3 connects to GND and V1(−). Make sure Node A and Node B each have a visible junction dot where wires meet.

- Place the Virtual Oscilloscope from the left toolbar. Connect the probes like this: Ch A → V1 positive terminal (input signal). Ch B → Node A (junction of R1 and R2 top). Ch C → Node B (junction of R2 bottom and R3 right). Connect all oscilloscope GND pins to circuit GND.

- Run the simulation by pressing Play. For 200 Hz, set Time/div to 1 ms/div to see full cycles. Then read off the voltages: V_R1 = ChA − ChB, V_R2 = ChB − ChC, V_R3 = ChC, V_(R1+R2) = ChA − ChC, V_(R2+R3) = ChB.

Expected Results for Circuit B

If all three resistors are effectively in series around the loop (from the Figure 7 topology):

Total resistance = 4.7k + 3.3k + 5.1k = 13.1 kΩ

V_R1 (peak) = 3.5 × (4700 / 13100) ≈ 1.26 V   →   Vpp ≈ 2.51 V

V_R2 (peak) = 3.5 × (3300 / 13100) ≈ 0.88 V   →   Vpp ≈ 1.76 V

V_R3 (peak) = 3.5 × (5100 / 13100) ≈ 1.36 V   →   Vpp ≈ 2.73 V

RMS = peak / √2 for each of the above.

Things to Be Careful About

- In Proteus, VSINE takes peak (amplitude) voltage — NOT peak-to-peak. Always divide Vpp by 2 before entering it.

- Make sure ground is connected properly, otherwise the simulation will give 0 V everywhere.

- When reading voltages from the oscilloscope, individual channels show voltage relative to GND. To get the voltage across a resistor that is not directly connected to GND, you need to subtract two channel readings.

- For resistor values, you can type '5.1k' or '5100' — both work in Proteus.

- RMS voltage for a sine wave = peak voltage / 1.414. The Proteus oscilloscope does not directly show RMS, so you calculate it manually from the peak reading.

Conclusion

Both Circuit A and Circuit B from the lab manual can be easily simulated in Proteus 8. The software gives you waveforms that are very close to what you would measure on a real oscilloscope in the lab, which helps confirm whether your theoretical calculations are correct. The main things to remember are to set the VSINE amplitude as half the peak-to-peak value, connect ground properly, and use multiple oscilloscope channels to measure voltages across individual resistors in the circuit.