Circuit Analysis Lab #3

Constructing Circuits in Proteus 8 with Virtual Multimeter

Experiment 3 — Introduction to Multimeter and Basic Series Circuit Implementation

Objective

This report explains how to build and simulate the two circuits from Lab #3 in Proteus 8. The main focus of this lab is using a multimeter to measure voltage, current, and resistance in series circuits. Since we are doing this in Proteus, we will use the virtual multimeter (DC Voltmeter and DC Ammeter) that comes built into the software instead of a physical DMM.

Multimeter in Proteus 8

In Proteus, there is no single 'multimeter' instrument like the physical DMM. Instead, it uses separate virtual instruments for each measurement:

- DC Voltmeter — measures voltage across a component (connected in parallel, just like a real voltmeter)

- DC Ammeter — measures current through a branch (connected in series, just like a real ammeter)

- You can also just hover over a wire during simulation and Proteus shows the voltage/current on that net

To add these instruments, click the 'Virtual Instruments Mode' button on the left toolbar and select DC Voltmeter or DC Ammeter from the list. Place them on the canvas and wire them into the circuit.

One important thing to remember — the ammeter must always be placed in series (in line with the wire), never across a component. If you accidentally connect it in parallel, it acts like a short circuit and will give wrong results. This is the same rule as with a real DMM.

Task 1 — Circuit with R1 = 5.1 kΩ and R2 = 3.3 kΩ (5V Source)

Circuit Description

This is the same series circuit from Lab #2. V1 is a 5V DC source connected to R1 (5.1 kΩ) and R2 (3.3 kΩ) in series. This time instead of using an oscilloscope, we use the multimeter (voltmeter and ammeter) to measure voltage across R1, voltage across R2, and the current flowing between them.

| Component | Value | Notes |
| --- | --- | --- |
| V1 (DC Voltage Source) | 5 V DC | Use VSOURCE or battery in Proteus |
| R1 | 5.1 kΩ | Series resistor |
| R2 | 3.3 kΩ | Series resistor |
| DC Voltmeter x2 | Virtual instrument | One across R1, one across R2 |
| DC Ammeter x1 | Virtual instrument | Placed in series between R1 and R2 |

How to Build Task 1 in Proteus

- Open Proteus 8 and create a new project. Name it something like 'Lab3_Task1'.

- Press P and search for 'VSOURCE'. Place it on the left side of the canvas. Double-click and set the voltage to 5V DC. Make sure it is set to DC (not sinusoidal this time).

- Press P again and search for 'RES'. Place two resistors — set R1 = 5.1k and R2 = 3.3k. Label them properly. Arrange them so R1 is at the top and R2 is on the right side, similar to the circuit diagram in the lab manual.

- Add a GROUND terminal from the Terminals Mode on the left toolbar. Connect it to the negative terminal of V1 and the bottom of R2.

- Press W and wire the circuit: V1(+) → R1 → midpoint node → R2 → GND → V1(−). This is the basic series loop.

- Now add the virtual voltmeters. Go to Virtual Instruments Mode and select DC Voltmeter. Place the first voltmeter and connect it across R1 (positive probe to top of R1, negative probe to bottom of R1). Place the second voltmeter across R2 in the same way.

- Add the DC Ammeter. Place it in series in the wire between R1 and R2 (at the midpoint node). To do this: break the wire at that point and insert the ammeter so current flows through it. The current direction should go from R1 into the ammeter and out to R2.

- Press the Play button to run the simulation. The voltmeter displays will show the voltage across each resistor, and the ammeter will show the current in the branch.

- Record the readings and compare with the theoretical values calculated below.

Theoretical Values for Task 1

Since this is a DC series circuit, we just use Ohm's Law:

Total resistance = R1 + R2 = 5.1k + 3.3k = 8.4 kΩ

Current I = V / R = 5 / 8400 ≈ 0.595 mA

Voltage across R1 = I × R1 = 0.000595 × 5100 ≈ 3.03 V

Voltage across R2 = I × R2 = 0.000595 × 3300 ≈ 1.96 V

Check: 3.03 + 1.96 = 4.99 V ≈ 5 V ✓

Task 2 — Circuit with R1 = 3 kΩ, R2 = 10 kΩ, R3 = 5 kΩ (9V Source)

Circuit Description

This circuit has a 9V DC source and three resistors all connected in a series loop. Looking at the circuit diagram in the lab manual (Figure with nodes 1, 2, 3, 4): R1 = 3 kΩ is at the top, R2 = 10 kΩ is on the right side (vertical), and R3 = 5 kΩ is at the bottom. Node 1 is the positive terminal, node 2 is between R1 and R2, node 3 is between R2 and R3, and node 4 is the negative terminal (GND). We need to measure voltage across each resistor and current at node 2 and node 3.

| Component | Value | Notes |
| --- | --- | --- |
| V1 (DC Voltage Source) | 9 V DC | Use VSOURCE in Proteus |
| R1 | 3 kΩ | Top horizontal resistor |
| R2 | 10 kΩ | Right vertical resistor |
| R3 | 5 kΩ | Bottom horizontal resistor |
| DC Voltmeter x3 | Virtual instrument | One across each resistor |
| DC Ammeter x2 | Virtual instrument | One at node 2, one at node 3 |

How to Build Task 2 in Proteus

- Create a new project or new sheet in Proteus. Name it 'Lab3_Task2'.

- Press P and place a VSOURCE. Set it to 9V DC. Place it on the left side vertically with + at the top.

- Add three resistors using P → RES. Set R1 = 3k (horizontal, top), R2 = 10k (vertical, right side — rotate it), R3 = 5k (horizontal, bottom). Label all three.

- Add a GROUND terminal and connect it to the negative of V1 and the left end of R3 (node 4).

- Wire the circuit to form a rectangular loop: V1(+) at node 1 → R1 → node 2 (top-right corner) → R2 (going down) → node 3 (bottom-right corner) → R3 → node 4 (GND) → V1(−). All four nodes should be clearly visible as junctions.

- Add three DC Voltmeters — one across R1 (nodes 1 to 2), one across R2 (nodes 2 to 3), one across R3 (nodes 3 to 4/GND). Connect positive probe to the higher-potential node each time.

- Add two DC Ammeters. Place the first ammeter in series at node 2 (break the wire between R1 and R2 and insert it there). Place the second ammeter in series at node 3 (break the wire between R2 and R3 and insert it there). Since this is a pure series circuit, both ammeters should read the same current — this confirms KCL.

- Run the simulation by pressing Play. All five instruments will display readings at the same time. Record the values into the practical values table in your lab sheet.

Theoretical Values for Task 2

Again using Ohm's Law for a series circuit:

Total resistance = R1 + R2 + R3 = 3k + 10k + 5k = 18 kΩ

Current I = V / R = 9 / 18000 = 0.5 mA

Voltage across R1 = 0.0005 × 3000 = 1.5 V

Voltage across R2 = 0.0005 × 10000 = 5.0 V

Voltage across R3 = 0.0005 × 5000 = 2.5 V

Check: 1.5 + 5.0 + 2.5 = 9.0 V ✓

Current at node 2 = 0.5 mA   |   Current at node 3 = 0.5 mA (same, since series circuit)

Things to Keep in Mind

- Always use VSOURCE (not VSINE) for DC circuits. VSINE is for AC signals.

- The voltmeter is connected in parallel across a component — it should not break the wire.

- The ammeter is connected in series — you must break the wire and insert it in between.

- If the ammeter shows a negative value, swap its probe connections (red and black are reversed).

- In a series circuit, the current is the same at every point, so node 2 and node 3 currents in Task 2 should be identical. If they are different in your simulation, check your wiring.

- Make sure all resistor values are entered correctly — '3k' means 3000 Ω, '10k' means 10000 Ω.

Conclusion

Both circuits from Lab #3 can be simulated in Proteus 8 using the virtual DC Voltmeter and DC Ammeter instruments. The simulated values should match the theoretical values calculated using Ohm's Law quite closely. This lab helped in understanding how to properly connect a voltmeter (parallel) and ammeter (series) to take measurements, which is an important concept not just for simulation but also for real lab work with an actual DMM.