Circuit Analysis Lab #4

Constructing Parallel Circuits in Proteus 8

Experiment 4 — Implementation of Basic Parallel Circuit

Objective

This report explains how to build and simulate the two parallel circuits from Lab #4 in Proteus 8. The main difference between this lab and the previous one is that the resistors are now connected in parallel instead of series. In a parallel circuit, each resistor gets the same voltage but carries a different current depending on its resistance value. We will use the virtual DC Ammeter and DC Voltmeter in Proteus to measure these values.

Parallel Circuit vs Series Circuit — Quick Reminder

Before getting into the Proteus steps, it helps to understand what makes a parallel circuit different from a series one:

- In a series circuit, the same current flows through all resistors and the voltage splits across them.

- In a parallel circuit, the same voltage appears across all resistors but the current splits into separate branches.

- Total resistance in parallel is always less than the smallest individual resistor.

- Formula: 1/RT = 1/R1 + 1/R2 + 1/R3 ...

This is important to keep in mind when wiring the circuit in Proteus, because each resistor needs its own separate branch connected between the same two nodes (top rail and bottom rail).

Task 1 — Three Parallel Resistors (9V Source, Nodes 1-8)

Circuit Description

Looking at the circuit diagram in the lab manual, there is a 9V DC source on the left side. Three resistors R1 = 10 kΩ, R2 = 2 kΩ, and R3 = 1 kΩ are connected in parallel between the top rail (nodes 1-2-3-4) and the bottom rail (nodes 8-7-6-5). Each resistor connects vertically between the top and bottom rails. We need to measure the current through each resistor and the voltage between node 2 and node 7.

| Component | Value | Notes |
| --- | --- | --- |
| V1 (DC Source) | 9 V DC | Use VSOURCE in Proteus |
| R1 | 10 kΩ | First parallel branch |
| R2 | 2 kΩ | Second parallel branch |
| R3 | 1 kΩ | Third parallel branch |
| DC Ammeter x3 | Virtual instrument | One in series with each resistor branch |
| DC Voltmeter x1 | Virtual instrument | Connected between node 2 and node 7 |

How to Build Task 1 in Proteus

- Open Proteus 8 and create a new project. Name it 'Lab4_Task1'.

- Press P and search for VSOURCE. Place it on the left side of the canvas vertically. Double-click and set the voltage to 9V DC.

- Press P and search for RES. Place three resistors vertically on the canvas, side by side. Set R1 = 10k, R2 = 2k, R3 = 1k and label them. Space them out a bit so there is room to add ammeters in each branch.

- Add a GROUND terminal from the Terminals Mode. Place it at the bottom connecting the negative of V1 and the bottom of all three resistors.

- Now wire the circuit. Draw a horizontal wire along the top connecting: V1(+) → top of R1 → top of R2 → top of R3. This is the top rail (nodes 1 to 4). Then draw a horizontal wire along the bottom connecting: V1(−) / GND → bottom of R1 → bottom of R2 → bottom of R3. This is the bottom rail (nodes 8 to 5). All three resistors are now in parallel.

- Add three DC Ammeters from the Virtual Instruments Mode. For each resistor, break the wire in that branch (either above or below the resistor) and insert the ammeter in series. So each branch has: top rail → ammeter → resistor → bottom rail (or top rail → resistor → ammeter → bottom rail, either is fine).

- Add one DC Voltmeter. Connect its positive probe to the top rail at the position of node 2 (between V1 and R1) and its negative probe to the bottom rail at node 7 (below R1). Since all top rail nodes are connected and all bottom rail nodes are connected, the voltage between node 2 and node 7 is basically the full supply voltage across R1.

- Press Play to run the simulation. All three ammeters will show different currents because the resistors have different values, but the voltmeter will show 9V across all of them (same voltage in parallel). Record the readings.

Theoretical Values for Task 1

Since all resistors have the same 9V across them, we just use Ohm's Law for each branch separately:

Current through R1 = V / R1 = 9 / 10000 = 0.9 mA

Current through R2 = V / R2 = 9 / 2000 = 4.5 mA

Current through R3 = V / R3 = 9 / 1000 = 9.0 mA

Voltage between node 2 and node 7 = 9 V (same as supply, since they are on the same top and bottom rails)

Total current from source = 0.9 + 4.5 + 9.0 = 14.4 mA

Task 2 — Four Parallel Resistors (9V Source, Nodes a to e)

Circuit Description

This circuit also has a 9V source but now there are four resistors all in parallel between node a (left) and node e (right). From the diagram: R1 = 100 Ω, R2 = 3 kΩ, R3 = 5 kΩ, R4 = 10 kΩ. The currents through each branch are labeled I1, I2, I3, I4 and the total current from the source is labeled I. We need to measure the individual branch currents and the voltage between node a and node e.

| Component | Value | Notes |
| --- | --- | --- |
| V1 (DC Source) | 9 V DC | Use VSOURCE in Proteus |
| R1 | 100 Ω | First parallel branch — note: ohms, not kilohms |
| R2 | 3 kΩ | Second parallel branch |
| R3 | 5 kΩ | Third parallel branch |
| R4 | 10 kΩ | Fourth parallel branch |
| DC Ammeter x4 | Virtual instrument | One per branch |
| DC Voltmeter x1 | Virtual instrument | Between node a and node e |

How to Build Task 2 in Proteus

- Create a new project or sheet and name it 'Lab4_Task2'.

- Press P, place a VSOURCE and set it to 9V DC. This time the source is at the bottom of the circuit based on the diagram, but you can place it on the left side in Proteus for convenience.

- Press P and add four resistors. Set: R1 = 100 (just type 100, no 'k'), R2 = 3k, R3 = 5k, R4 = 10k. Label all four. Arrange them side by side vertically between two horizontal rails.

- Add a GROUND terminal and connect it to the negative terminal of V1.

- Wire the top rail: V1(+) → node a → connects to top of all four resistors (R1, R2, R3, R4). Wire the bottom rail: bottom of all four resistors → node e → back to V1(−)/GND. All four resistors share the same two nodes (a and e) which means they are all in parallel.

- Add four DC Ammeters, one in series with each resistor branch. Break each branch wire and insert the ammeter so current flows through it. Label the ammeters I1 through I4 to keep track.

- Add one DC Voltmeter between node a (top rail) and node e (bottom rail). This should read 9V since those are directly connected to the supply terminals.

- Run the simulation. You will notice R1 (100 Ω) carries a much larger current than the others because it has the lowest resistance. This is an important observation about parallel circuits — the branch with the lowest resistance takes the most current.

Theoretical Values for Task 2

Again, each resistor has 9V across it so we calculate each branch current separately:

Current through R1 = 9 / 100 = 90 mA

Current through R2 = 9 / 3000 = 3 mA

Current through R3 = 9 / 5000 = 1.8 mA

Current through R4 = 9 / 10000 = 0.9 mA

Voltage between node a and node e = 9 V

Total current I = 90 + 3 + 1.8 + 0.9 = 95.7 mA

Notice how R1 = 100 Ω dominates the total current because it is so much smaller than the others. This is a good example of how the lowest resistance branch draws the most current in a parallel circuit.

Things to Keep in Mind

- In a parallel circuit, every resistor connects between the same two nodes. If you accidentally wire them in series instead, the voltages and currents will be completely wrong.

- The voltage across every parallel branch is always equal to the source voltage (9V in both tasks). So the voltmeter reading should always match the supply.

- Each ammeter must go in series inside its own branch. Do not share one ammeter between two branches.

- R1 in Task 2 is 100 ohms — make sure you type '100' and not '100k' by mistake. A wrong value will mess up all the current readings.

- After simulation, you can verify your results using KCL: the sum of all branch currents should equal the total current supplied by the source.

Conclusion

Both parallel circuits from Lab #4 were successfully built and simulated in Proteus 8 using the virtual DC Ammeter and DC Voltmeter. The key thing this lab demonstrates is that in a parallel circuit, voltage stays the same across all branches while the current divides. The branch with the smallest resistance always carries the most current, which is clearly seen in Task 2 where R1 = 100 Ω draws 90 mA compared to the few milliamps in the other branches. The simulated values should match the theoretical Ohm's Law calculations very closely.