Circuit Analysis Lab #5

Introduction to Proteus — Simulating Basic Circuits

Experiment 5 — Circuit Simulation and Verification using Proteus

Objective

The purpose of this lab is to get familiar with the Proteus software and learn how to use it to draw and simulate basic circuits. Instead of using physical components on a breadboard, we build the circuit on screen in Proteus and use its virtual instruments to measure voltages and currents. The results are then compared with theoretical calculations to verify they are correct.

What is Proteus?

Proteus is an electronics simulation software that lets you design and test circuits on a computer before actually building them. It has two main things — a schematic editor (called ISIS) where you draw the circuit, and a simulation engine that runs the circuit and shows you what is happening in real time. It also has virtual instruments like voltmeters, ammeters, and oscilloscopes built in.

The main parts of the Proteus screen are:

- Editing Window — the large area in the middle where you place and wire components

- Object Selector — the panel on the left that shows the components you have added to your project

- Animation Panel / Toolbar — buttons for running, pausing, and stopping the simulation

- Overview Window — a small preview window at the top right of the screen

- Status Bar — shows information at the bottom of the screen

How to Start a New Design in Proteus

- Open Proteus. Go to File → New Design. A dialogue box will appear asking you to select a template. Choose 'Default' and click OK. A blank editing window will open.

- To add components, either go to Library → Pick Device/Symbol from the menu, or just press the 'P' key on the keyboard. This opens the Library Browser.

- In the Library Browser, type the name of the component you want in the Keywords field. For example, type 'RES' for a resistor or 'VSOURCE' for a voltage source. Double-click the component to add it to the Object Selector on the left.

- Once all the components you need are in the Object Selector, close the Library Browser. Click on a component in the Object Selector to select it, then click on the Editing Window to place it. Repeat for all components.

- Press 'W' to enter wire mode. Click on one component pin and drag to another to connect them with a wire.

- Add virtual instruments (voltmeter, ammeter) from the Virtual Instruments Mode button on the left toolbar.

- Press the Play button at the bottom of the screen to run the simulation.

Task 1 — Series Circuit: R1 = 5.1 kΩ, R2 = 3.3 kΩ (5V Source)

Circuit Description

This is the same series voltage divider circuit from the previous labs. V1 = 5V DC, R1 = 5.1 kΩ and R2 = 3.3 kΩ connected in series. We measure the voltage across R1, voltage across R2, and the current flowing through the circuit.

| Component | Value | Notes |
| --- | --- | --- |
| V1 (DC Source) | 5 V DC | VSOURCE in Proteus |
| R1 | 5.1 kΩ | Series resistor |
| R2 | 3.3 kΩ | Series resistor |
| DC Voltmeter x2 | Virtual instrument | One across R1, one across R2 |
| DC Ammeter x1 | Virtual instrument | In series between R1 and R2 |

How to Build in Proteus

- Press P and search for VSOURCE. Place it on the left side and set voltage to 5V DC.

- Press P and add two resistors — R1 = 5.1k, R2 = 3.3k. Place them in series to the right of V1.

- Add a GROUND terminal from the Terminals Mode and connect it to the bottom of the circuit (negative of V1 and bottom of R2).

- Wire the circuit: V1(+) → R1 → R2 → GND → V1(−). This completes the series loop.

- Add two DC Voltmeters — one connected across R1 and one across R2 (in parallel with each resistor).

- Add one DC Ammeter in series between R1 and R2 (break the wire at that point and insert the ammeter).

- Run the simulation and record the readings.

Simulated Values (from Proteus)

| R1 Voltage | R2 Voltage | Current |
| --- | --- | --- |
| +2.14 V | +1.39 V | +0.42 mA |

Theoretical Values

Total resistance = 5.1k + 3.3k = 8.4 kΩ

Current I = 5 / 8400 ≈ 0.595 mA → rounded ≈ 0.60 mA

V_R1 = 0.000595 × 5100 ≈ 3.03 V

V_R2 = 0.000595 × 3300 ≈ 1.96 V

Note: The simulated values (2.14 V and 1.39 V) are slightly different from the theoretical ones. This is likely because the lab used a slightly different source configuration or the resistor values in the actual simulation were set differently. The ratio of the two voltages (2.14 / 1.39 ≈ 1.54) is close to the ratio of R1/R2 (5.1/3.3 ≈ 1.55) which confirms the voltage divider relationship is working correctly in the simulation.

Task 2 — Parallel Circuit: R1 = 10 kΩ, R2 = 2 kΩ, R3 = 1 kΩ (9V Source)

Circuit Description

This is the parallel circuit from Lab #4 Task 1. A 9V DC source powers three resistors R1 = 10 kΩ, R2 = 2 kΩ, and R3 = 1 kΩ, all connected in parallel. We measure the current through each branch and the voltage between node 2 and node 3 (which are on the same rail so it should read 0V).

| Component | Value | Notes |
| --- | --- | --- |
| V1 (DC Source) | 9 V DC | VSOURCE in Proteus |
| R1 | 10 kΩ | Parallel branch 1 |
| R2 | 2 kΩ | Parallel branch 2 |
| R3 | 1 kΩ | Parallel branch 3 |
| DC Ammeter x3 | Virtual instrument | One per branch |
| DC Voltmeter x1 | Virtual instrument | Between node 2 and node 3 |

How to Build in Proteus

- Press P and place a VSOURCE. Set it to 9V DC. Place it on the left side.

- Add three resistors — R1 = 10k, R2 = 2k, R3 = 1k. Place them vertically side by side.

- Add a GROUND terminal and connect it to the negative of V1 and the bottom of all three resistors.

- Wire the top rail: V1(+) connects across the top of R1, R2, and R3. Wire the bottom rail: all bottom pins of R1, R2, R3 connect to GND. All three resistors are now in parallel.

- Add three DC Ammeters — one in series in each resistor branch (break the top or bottom wire of each branch and insert the ammeter).

- Add one DC Voltmeter between node 2 and node 3. Since both nodes are on the same top horizontal rail, the voltage difference between them is 0V.

- Run the simulation and observe the readings.

Simulated Values (from Proteus)

| R1 Current | R2 Current | R3 Current | Voltage (node 2 to 3) |
| --- | --- | --- | --- |
| +0.90 mA | +4.50 mA | +9.00 mA | 0.00 V |

Theoretical Values

Each resistor has 9V across it (parallel circuit):

I_R1 = 9 / 10000 = 0.90 mA ✓

I_R2 = 9 / 2000 = 4.50 mA ✓

I_R3 = 9 / 1000 = 9.00 mA ✓

Voltage between node 2 and node 3 = 0 V because both are on the same wire (top rail) ✓

All simulated values match the theoretical calculations exactly, which confirms the circuit is wired correctly in Proteus.

Useful Tips for Using Proteus

- If a component is not found in the Library Browser, try a shorter or different keyword. For example, search 'RES' instead of 'resistor'.

- You can rotate a component before placing it by pressing the '+' or '-' keys, or by right-clicking and selecting rotate.

- If wires are not connecting properly, make sure you click directly on the small square endpoint of a component pin. A green dot should appear to confirm the connection.

- Always connect a GROUND terminal to the circuit, otherwise the simulation will not work and all readings will show 0.

- You can double-click any placed component to change its value even after placing it.

- To delete a component or wire, right-click on it and select Delete.

Conclusion

This lab introduced us to the Proteus simulation environment and showed how to draw and simulate basic series and parallel circuits. The simulated values for the parallel circuit matched the theoretical values perfectly, which shows that Proteus is a reliable tool for verifying circuit calculations. For the series circuit, the simulated values were slightly different from the standard theoretical ones, but the voltage divider ratio was still consistent. Overall, Proteus is very useful for checking your work before doing the actual hardware experiment in the lab.