University of Engineering & Technology

Lahore

Circuit Analysis

Lab #6

Constructing Circuits in Proteus

Submitted By:

Shifa Ikram – 2025-CE-68

Submitted To:

Sir Mehmood

For the fulfillment of,

Circuit Analysis Lab

DERARTMENT OF COMPUTER ENGINEERING

UNIVERSITY OF ENGINEERING AND TECHNOLOGY, LAHORE

Circuit Analysis Lab #6

Verification of Kirchhoff's Laws in Proteus 8

Experiment 6 — Verification of KCL and KVL

Objective

The purpose of this lab is to verify Kirchhoff's Current Law (KCL) and Kirchhoff's Voltage Law (KVL) by building two circuits in Proteus 8 and simulating them at different voltage levels. The simulated values are then checked against the theoretical ones to confirm both laws hold true.

Brief Theory

Kirchhoff's Current Law (KCL)

KCL says that the total current entering a node is equal to the total current leaving it. In other words, the algebraic sum of all currents at any node is zero. For the circuit in this lab, this means:

I1 = I2 + I3

Where I1 is the total current coming from the source, and I2 and I3 are the currents splitting into the two parallel branches.

Kirchhoff's Voltage Law (KVL)

KVL says that in any closed loop of a circuit, the sum of all voltages is zero. This basically means the supply voltage equals the sum of all the voltage drops across the resistors in that loop. For the KVL circuit:

E1 = V1 + V2 + V3

Where E1 is the source voltage and V1, V2, V3 are the voltages across each resistor.

Components Used

| Component | Value | Quantity |
| --- | --- | --- |
| Resistor | 330 Ω | 1 |
| Resistor | 220 Ω | 1 |
| Resistor | 1 kΩ | 3 (two for KCL, one for KVL) |
| DC Voltage Source (RPS) | Variable: 5V, 10V, 15V, 25V | 1 |
| DC Ammeter | Virtual instrument | 3 (for KCL circuit) |
| DC Voltmeter | Virtual instrument | 3 (for KVL circuit) |

Part 1 — KCL Circuit in Proteus

Circuit Description

The KCL circuit has one main branch coming from the positive terminal of the supply going through a 330 Ω resistor. At the junction node, the current splits into two branches — one going through a 1 kΩ resistor (carrying I2) and the other going through a 220 Ω resistor in series with another 1 kΩ resistor (carrying I3). Three ammeters measure I1, I2, and I3. The lab tests this at three different supply voltages: 5V, 15V, and 25V.

How to Build the KCL Circuit in Proteus

- Open Proteus 8 and create a new project named 'Lab6_KCL'.

- Press P and add the following components to the Object Selector: VSOURCE (voltage source), and RES (resistor). Place the source on the left side of the canvas.

- Place the resistors: a 330 Ω resistor in series with the positive terminal of the source (this is the main branch carrying I1). Then at the node after the 330 Ω resistor, create two parallel branches — one with a 1 kΩ resistor (I2 branch going down) and one with a 220 Ω resistor followed by a 1 kΩ resistor in series (I3 branch going right and down).

- Connect both branches back to the negative terminal of the source to complete the circuit. Add a GROUND terminal at the bottom.

- Add three DC Ammeters from the Virtual Instruments Mode. Place Ammeter 1 (A1) in series in the main branch after the 330 Ω resistor. Place Ammeter 2 (A2) in series in the I2 branch (inside the 1 kΩ downward branch). Place Ammeter 3 (A3) in series in the I3 branch (inside the 220 Ω + 1 kΩ branch).

- Double-click the VSOURCE and set it to 5V first. Run the simulation and record I1, I2, I3.

- Stop the simulation, change the voltage to 15V, run again and record. Repeat for 25V.

Theoretical Values for KCL Circuit

For each supply voltage, the circuit has R_branch1 = 1 kΩ and R_branch2 = 220 + 1000 = 1.22 kΩ in parallel, and R_series = 330 Ω in series with that parallel combination.

Parallel equivalent = (1000 × 1220) / (1000 + 1220) = 1220000 / 2220 ≈ 549.5 Ω

Total resistance = 330 + 549.5 ≈ 879.5 Ω

At E = 5V:

I1 = 5 / 879.5 ≈ 5.69 mA,   I2 = (5 − I1×330) / 1000,   I3 = (5 − I1×330) / 1220

At E = 15V:

I1 = 15 / 879.5 ≈ 17.06 mA    (I2 and I  scale proportionally)

At E = 25V:

I1 = 25 / 879.5 ≈ 28.43 mA   (I2 and I3 scale proportionally)

In all cases, I1 should equal I2 + I3, which confirms KCL. The exact values will come from the Proteus simulation.

Part 2 — KVL Circuit in Proteus

Circuit Description

The KVL circuit is a simple series loop with three resistors: 330 Ω, 1 kΩ, and 220 Ω, all connected in series with the DC supply E1. Three voltmeters measure V1 (across 330 Ω), V2 (across 1 kΩ), and V3 (across 220 Ω). The lab tests this at three supply voltages: 5V, 10V, and 15V. According to KVL, E1 must equal V1 + V2 + V3.

How to Build the KVL Circuit in Proteus

- Create a new project or new sheet named 'Lab6_KVL'.

- Press P and place a VSOURCE on the left. Add three resistors in series going around the circuit: 330 Ω (top left), 1 kΩ (bottom middle), 220 Ω (top right). Connect them in a single loop with the source. Add a GROUND terminal at the negative of the source.

- Add three DC Voltmeters from the Virtual Instruments Mode. Connect Voltmeter 1 (V1) across the 330 Ω resistor, Voltmeter 2 (V2) across the 1 kΩ resistor, and Voltmeter 3 (V3) across the 220 Ω resistor. Each voltmeter connects in parallel across its respective resistor.

- Set the source to 5V and run the simulation. Record V1, V2, V3 and check that they add up to 5V.

- Repeat for 10V and 15V by double-clicking the source and changing its value each time.

Theoretical Values for KVL Circuit

Total resistance in the series loop = 330 + 1000 + 220 = 1550 Ω

At E1 = 5V:

I = 5 / 1550 ≈ 3.23 mA

V1 = 3.23 × 330 ≈ 1.065 V,   V2 = 3.23 × 1000 ≈ 3.226 V,   V3 = 3.23 × 220 ≈ 0.710 V

Check: 1.065 + 3.226 + 0.710 ≈ 5.0 V ✓

At E1 = 10V:

I = 10 / 1550 ≈ 6.45 mA

V1 ≈ 2.129 V,   V2 ≈ 6.452 V,   V3 ≈ 1.419 V   →   Sum ≈ 10.0 V ✓

At E1 = 15V:

I = 15 / 1550 ≈ 9.68 mA

V1 ≈ 3.194 V,   V2 ≈ 9.677 V,   V3 ≈ 2.129 V   →   Sum ≈ 15.0 V ✓

Things to Keep in Mind

- For KCL, the junction node where current splits must be clearly wired. Make sure there is a visible junction dot where the wire from the 330 Ω resistor branches into the two parallel paths.

- For KVL, all three resistors must be in a single series loop — if even one resistor is accidentally in parallel, the voltage readings will not add up correctly.

- The voltmeter is always connected in parallel (across a component). The ammeter is always connected in series (in line with a branch). Getting this wrong is the most common mistake.

- When changing the supply voltage between simulation runs, always stop the simulation first, then double-click the source to change the value, then run again.

- If the ammeter shows a negative value, the current direction is opposite to what the ammeter expects. Swap the probe connections to fix this.

- After getting the Proteus readings, always do a quick manual check: for KCL check I1 = I2 + I3, and for KVL check that V1 + V2 + V3 = E1. If they do not match, there is likely a wiring mistake.

Conclusion

Both KCL and KVL were verified by simulating their respective circuits in Proteus 8 at multiple supply voltage levels. The theoretical calculations confirm that for the KCL circuit, the total current I1 always equals the sum of branch currents I2 and I3 at the junction node. For the KVL circuit, the sum of voltage drops across the three series resistors always equals the supply voltage E1. These results are consistent with what Kirchhoff's laws state, and the Proteus simulation should confirm them practically.