Circuit Analysis Lab #7

Inverting and Non-Inverting Op-Amp Circuits in Proteus

Experiment 7 — Analysis of Operational Amplifier Operations

Objective

This lab is about building and simulating inverting and non-inverting amplifier circuits using the 741 op-amp in Proteus. The goal is to observe the input and output waveforms on the virtual oscilloscope and verify the gain for both configurations. The input is a 2 V peak-to-peak, 40 Hz sine wave.

Brief Theory

What is an Op-Amp?

An operational amplifier (op-amp) is an IC that amplifies the difference between two input voltages. The 741 is one of the most commonly used op-amps. It needs a dual power supply (positive and negative VCC) to operate. In this lab we use it in two configurations — inverting and non-inverting.

Inverting Amplifier

In the inverting amplifier, the input signal is fed into the inverting terminal (pin 2, marked with a minus sign) through a resistor R1. A feedback resistor R2 connects the output back to pin 2. The non-inverting terminal (pin 3) is connected to ground. The output is 180 degrees out of phase with the input — meaning when the input goes up, the output goes down and vice versa.

The gain formula for the inverting amplifier is:

Gain (Av) = −R2 / R1

The negative sign means the output is inverted. For this lab: R1 = 1 kΩ, R2 = 3 kΩ, so Av = −3. This means the output is 3 times larger than the input and flipped.

Non-Inverting Amplifier

In the non-inverting amplifier, the input signal goes into the non-inverting terminal (pin 3, marked with a plus sign) directly. R1 connects from the inverting terminal (pin 2) to ground, and R2 is the feedback resistor from the output back to pin 2. The output is in phase with the input — it looks the same but larger.

The gain formula for the non-inverting amplifier is:

Gain (Av) = 1 + (R2 / R1)

For this lab: R1 = 1 kΩ, R2 = 2 kΩ, so Av = 1 + 2 = 3. The output is 3 times larger than the input with no phase flip.

IC 741 Op-Amp Pin Configuration

Before building the circuit in Proteus, it helps to know the 741 op-amp pin layout since Proteus shows the actual IC with numbered pins:

| Pin Number | Function |
| --- | --- |
| Pin 1 | Offset Null (leave unconnected for basic circuits) |
| Pin 2 | Inverting Input (−) |
| Pin 3 | Non-Inverting Input (+) |
| Pin 4 | Negative Power Supply (−VCC) |
| Pin 5 | Offset Null (leave unconnected) |
| Pin 6 | Output |
| Pin 7 | Positive Power Supply (+VCC) |
| Pin 8 | No Connection (NC) |

Part 1 — Inverting Amplifier in Proteus

Circuit Description

The inverting amplifier uses R1 = 1 kΩ (input resistor) and R2 = 3 kΩ (feedback resistor). The signal source V1 is a 2 Vpp, 40 Hz sine wave. The input goes through R1 into pin 2 of the 741. R2 connects pin 2 to pin 6 (output). Pin 3 is connected to ground. Pins 7 and 4 are connected to +VCC and −VCC respectively (typically ±12V or ±15V).

| Component | Value | Notes |
| --- | --- | --- |
| V1 (Signal Source) | 2 Vpp, 40 Hz, Sine | Use VSINE in Proteus — Amplitude = 1V |
| R1 (Input Resistor) | 1 kΩ | Connected between V1 and pin 2 |
| R2 (Feedback Resistor) | 3 kΩ | Connected between pin 6 and pin 2 |
| IC 741 Op-Amp | uA741 in Proteus | Search '741' in component library |
| +VCC Power Supply | +12V DC | Connected to pin 7 |
| −VCC Power Supply | −12V DC | Connected to pin 4 |
| Oscilloscope | Virtual (built-in) | Channel A = input, Channel B = output |

How to Build the Inverting Amplifier in Proteus

- Open Proteus 8 and create a new project named 'Lab7_Inverting'.

- Press P and search for '741' or 'uA741'. Select it and place it in the centre of the canvas. The op-amp symbol will appear with its pins labeled.

- Press P and add VSINE for the input source. Set Amplitude = 1V (which gives 2 Vpp), Frequency = 40 Hz. Place it on the left side.

- Add two resistors: R1 = 1k and R2 = 3k. Place R1 horizontally between V1 and pin 2 of the op-amp. Place R2 horizontally between pin 6 (output) and pin 2 — this is the feedback path.

- Connect pin 3 (non-inverting input) to GROUND using a Ground terminal from the Terminals Mode.

- Add two DC voltage sources for the power supply. One set to +12V connected to pin 7, and one set to −12V connected to pin 4. The negative supply needs a source with negative voltage — in Proteus you can use VSOURCE and set the value to −12. Connect the positive terminal of the +12V source to pin 7 and its negative to GND. For the −12V source, connect its negative terminal to pin 4 and its positive to GND.

- Add a GROUND terminal. Connect it to the negative of V1 input source, pin 3 of the op-amp, and the reference point of both power supplies.

- Add the Virtual Oscilloscope from the Virtual Instruments Mode. Connect Channel A probe to the input (at V1 positive terminal or the junction before R1). Connect Channel B probe to pin 6 (output). Connect oscilloscope GND to circuit GND.

- Run the simulation. You should see two sine waves on the oscilloscope. Channel A shows the original input (2 Vpp) and Channel B shows the inverted output (6 Vpp, flipped upside down). The output amplitude should be 3 times the input because Av = −R2/R1 = −3.

Expected Results — Inverting Amplifier

Input: 2 Vpp, 40 Hz sine wave

Gain = −R2/R1 = −3000/1000 = −3

Output: 2 × 3 = 6 Vpp, 40 Hz sine wave, but inverted (180° phase shift)

On the oscilloscope, when Channel A is at its positive peak, Channel B should be at its negative peak. This confirms the inverting behaviour.

Part 2 — Non-Inverting Amplifier in Proteus

Circuit Description

The non-inverting amplifier uses R1 = 1 kΩ and R2 = 2 kΩ. The input signal V1 (2 Vpp, 40 Hz) goes directly into pin 3 (non-inverting input). R1 connects pin 2 (inverting input) to ground. R2 is the feedback resistor connecting pin 6 (output) back to pin 2. The output is in phase with the input.

| Component | Value | Notes |
| --- | --- | --- |
| V1 (Signal Source) | 2 Vpp, 40 Hz, Sine | Amplitude = 1V in Proteus |
| R1 (Ground Resistor) | 1 kΩ | Connected between pin 2 and GND |
| R2 (Feedback Resistor) | 2 kΩ | Connected between pin 6 and pin 2 |
| IC 741 Op-Amp | uA741 in Proteus | Same component as before |
| +VCC Power Supply | +12V DC | Connected to pin 7 |
| −VCC Power Supply | −12V DC | Connected to pin 4 |
| Oscilloscope | Virtual (built-in) | Channel A = input, Channel B = output |

How to Build the Non-Inverting Amplifier in Proteus

- Create a new project or sheet named 'Lab7_NonInverting'.

- Place the uA741 op-amp on the canvas again (search '741' using P key).

- Place a VSINE source with Amplitude = 1V and Frequency = 40 Hz. This time connect it directly to pin 3 (non-inverting input) — no input resistor needed between the source and pin 3.

- Add R1 = 1k and connect it between pin 2 and GND. This is not a feedback resistor — it just sets a reference.

- Add R2 = 2k and connect it between pin 6 (output) and pin 2 (inverting input). This is the feedback path.

- Connect the +12V and −12V power supplies to pin 7 and pin 4 respectively, same as before.

- Connect GROUND to: the negative of V1, the bottom of R1, and the reference of both power supplies.

- Add the Virtual Oscilloscope. Connect Channel A to the junction at pin 3 (input signal). Connect Channel B to pin 6 (output). Run the simulation.

- This time both waveforms should be in phase — when Channel A goes up, Channel B goes up too. The output should be larger by a factor of 3 (Av = 1 + R2/R1 = 1 + 2 = 3), so output = 6 Vpp.

Expected Results — Non-Inverting Amplifier

Input: 2 Vpp, 40 Hz sine wave

Gain = 1 + R2/R1 = 1 + 2000/1000 = 3

Output: 2 × 3 = 6 Vpp, 40 Hz sine wave, same phase as input (no flip)

On the oscilloscope, Channel A and Channel B should look identical in shape, just Channel B will be 3 times taller. This is the key difference from the inverting amplifier.

Comparison Between Inverting and Non-Inverting Amplifier

| Feature | Inverting Amplifier | Non-Inverting Amplifier |
| --- | --- | --- |
| Input Connected To | Pin 2 (through R1) | Pin 3 (directly) |
| Phase of Output | 180° inverted | Same as input (0° shift) |
| Gain Formula | Av = −R2/R1 | Av = 1 + R2/R1 |
| Gain (this lab) | −3 (magnitude = 3) | +3 |
| Output Amplitude | 6 Vpp | 6 Vpp |
| R1 Role | Input resistor | Sets reference at pin 2 |
| R2 Role | Feedback resistor | Feedback resistor |

Things to Keep in Mind

- The op-amp must have both +VCC and −VCC power supplies connected (pins 7 and 4). If you forget to connect them, the output will stay at 0V and nothing will happen.

- In Proteus, search for '741' or 'uA741' to find the op-amp. Do not confuse it with other op-amp models.

- The VSINE amplitude is the peak value — for a 2 Vpp signal, set Amplitude = 1V in Proteus.

- For the inverting amplifier, pin 3 must be connected to GND. For the non-inverting amplifier, the input source goes to pin 3 and pin 2 connects to GND through R1.

- If the output is clipping (flat top or bottom), the output has exceeded the supply voltage rails (±12V). This would only happen if the gain is too high or the input is too large.

- To see the phase difference clearly, make sure both oscilloscope channels are set to the same Volts/div. Adjust Time/div so at least 2 cycles of the 40 Hz wave are visible — set Time/div to about 5 ms/div.

Conclusion

Both the inverting and non-inverting amplifier circuits were built and simulated in Proteus 8 using the 741 op-amp. The inverting amplifier with R1 = 1 kΩ and R2 = 3 kΩ gives a gain of −3, meaning the output is three times larger than the input and flipped. The non-inverting amplifier with R1 = 1 kΩ and R2 = 2 kΩ gives a gain of +3, meaning the output is three times larger but in the same phase as the input. The oscilloscope waveforms confirm these results and the gain formulas work as expected.